Là cổng kết nối trực tiếp đến máy người dùng (PC, laptop, máy in).
Access-порт — это порт, который напрямую подключается к пользовательскому устройству (ПК, ноутбук, принтер).

trunk port (cổng thân): Dùng để kết nối giữa các switch với nhau, hoặc giữa switch với router.

Câu lệnh kiểm tra: show vlan brief

Trunk-порт — это порт, используемый для соединения между коммутаторами или между коммутатором и маршрутизатором.

настроил

| Switch | Interface | Подключено к | Роль | VLAN |
| --- | --- | --- | --- | --- |
| IOU2 | e0/1 | PC1 | Access | VLAN 10 |
| IOU2 | e0/2 | PC2 | Access | VLAN 15 |
| IOU2 | e0/0 | IOU1 | Trunk | Все VLAN |
| IOU3 | e0/1 | PC3 | Access | VLAN 15 |
| IOU3 | e0/2 | PC4 | Access | VLAN 20 |
| IOU3 | e0/0 | IOU1 | Trunk | Все VLAN |
| IOU1 | e0/0 | R1 | Trunk | Все VLAN |
| IOU1 | e0/1 | IOU2 | Trunk | Все VLAN |
| IOU1 | e0/2 | IOU3 | Trunk | Все VLAN |

Tạo sub-interface trên R1

show ip interface brief

Cấu hình dhcp server:

show ip dhcp binding

Xóa port security:

conf t

interface Ethernet0/1

no switchport port-security

no switchport port-security maximum

no switchport port-security mac-address sticky

no switchport port-security violation

no spanning-tree portfast

interface Ethernet0/2

no switchport port-security

no switchport port-security maximum

no switchport port-security mac-address sticky

no switchport port-security violation

no spanning-tree portfast

exit

IOU3:

conf t

interface Ethernet0/1

shutdown

no switchport port-security

no switchport port-security maximum

no switchport port-security mac-address sticky

no switchport port-security violation

no spanning-tree portfast

no shutdown

interface Ethernet0/2

shutdown

no switchport port-security

no switchport port-security maximum

no switchport port-security mac-address sticky

no switchport port-security violation

no spanning-tree portfast

no shutdown

exit

kiểm tra sau khi gỡ: show run interface Ethernet0/1