# Overview

| Device | Port | IP /30 (+ /24 LAN) | Role |
| --- | --- | --- | --- |
| PC1-1 | e0 | 192.168.10.2/24 | Client A |
| Cisco1 | e1/1 | 192.168.10.1/24 | LAN A |
|  | e1/0 | 172.16.0.1/30 | WAN A |
| R1 | e0/0 | 172.16.0.2/30 | Connects to Cisco1 |
|  | e0/1 | 172.16.0.5/30 | Connects to pfSense |
| pfSense-1 | WAN | 172.16.0.6/30 | WAN B |
|  | LAN | 192.168.20.1/24 | LAN B |
| PC2-1 | e0 | 192.168.20.2/24 | Client B |

Phase 1 (ISAKMP)

Exchange type  Main mode

Authentication method  PSK

Encryption algorithm  AES-256-CTR

Authentication algorithm  SHA-256

Diffie-Hellman group  group 2

IKE session key lifetime  86400 s = 1 day

Phase 2 (IPSec)

IPSec Protocol  ESP

Mode  Tunnel

Encryption algorithm  AES-256-CTR

Authentication algorithm  HMAC-SHA-256

IPSec session key lifetime  3600 s = 1 hour

Perfect Forward Secrecy (PFS)  group 5

Network diagram:

# Network Lab Environment

- GNS3 và GNS3 VM: https://github.com/GNS3/gns3-gui/releases/tag/v3.0.5

- Router L3: https://github.com/obscur95/gns3-server/blob/master/IOU/i86bi-linux-l3-adventerprisek9-15.5.2T.bin

- iouLicense (Router L3) https://github.com/obscur95/gns3-server/blob/master/IOU/CiscoIOUKeygen.py

- Cisco: https://github.com/obscur95/gns3-server/blob/master/IOS/c7200-adventerprisek9-mz.152-4.S7.bin

- Ubuntu: https://ubuntu.com/download

- pfSense: https://www.pfsense.org/download/

# GNS3

Server:

IOS:

IOS:

add iourc file (licence)

router

PC:

# GNS3 VM

# PC1 - Virtual Machine

Install tools:

sudo apt update

sudo apt install -y net-tools iproute2 iputils-ping traceroute dnsutils curl wget

sudo apt install -y iperf3

sudo apt install -y tcpdump

sudo apt install -y openssh-server

IP configuration for PC1:

network:

version: 2

ethernets:

enp0s3:

dhcp4: no

addresses:

- 192.168.10.2/24

gateway4: 192.168.10.1

routes:

- to: 192.168.20.0/24

via: 192.168.10.1

# PC2 - Virtual Machine

Install tools:

sudo apt update

sudo apt install -y net-tools iproute2 iputils-ping traceroute dnsutils curl wget

sudo apt install -y iperf3

sudo apt install -y tcpdump

sudo apt install -y openssh-server

IP configuration for PC2:

network:

version: 2

ethernets:

enp0s3:

dhcp4: no

addresses:

- 192.168.20.2/24

gateway4: 192.168.20.1

routes:

- to: 192.168.10.0/24

via: 192.168.20.1

# pfSense